Smart Motor/Actuator Health Monitoring & Protection Node

Revision: 0.2
Current phase: Phase 1 — System Requirements
Status: Requirements draft; not yet baselined

1. Project Objective

Develop and experimentally validate a smart embedded node capable of:

- measuring DC motor/actuator current,

- identifying normal and abnormal operating behaviour,

- detecting overload and stall conditions,

- communicating measurements and diagnostic information,

- and ultimately protecting the actuator by commanding a shutdown.

The system shall be designed using professional requirements, analysis, implementation and laboratory-validation methods.

2. Intended Application

The demonstrator will monitor a low-voltage DC motor or actuator.

Typical operating states to distinguish:

- Motor OFF

- Startup/inrush

- Normal running

- Increased mechanical load

- Overload

- Stall/jam

- Electrical fault or abnormal measurement

The current waveform will therefore be treated not only as a measured quantity, but also as a source of diagnostic information.

3. High-Level Architecture

DC SUPPLY

│

├──────────── Motor / Actuator

│                 │

│             Load current

│                 ▼

│              ACS724

│                 │

│          Analog voltage

│                 ▼

│     Protection / Conditioning

│                 ▼

│        Anti-alias Filter

│                 ▼

│             MCU ADC

│                 │

│        ┌────────┴─────────┐

│        │                  │

│   Measurements       Diagnostics

│        │                  │

│        │            ┌─────┴─────┐

│        │            │           │

│        │         Warning      Fault

│        │                        │

│        └────────────┬───────────┘

│                     ▼

│             UART / RS-485 / CAN

│                     │

│                     ▼

│              PC / MATLAB

│

└── Future protection switch / MOSFET

4. Fixed Hardware Constraints

4.1 Current Sensor

Selected hardware:

Pololu #4048 / Allegro ACS724LLCTR-05AU

The selected device establishes:

- current measurement range: 0–5 A

- measurement direction: unidirectional

- sensor supply: approximately 5 V

- analog sensor output

- galvanically isolated Hall-effect current measurement

The 0–5 A range is therefore a hardware constraint rather than an arbitrary system requirement.

4.2 Controller

Selected controller:

Arduino UNO R4 WiFi

The MCU will initially perform:

- ADC acquisition

- timing

- filtering/processing

- calibration

- fault detection

- UART diagnostics

- CAN communication

RS-485 will use an external transceiver.

5. Functional Requirements

| ID | Requirement |
| --- | --- |
| FR-001 | The system shall measure motor/actuator current over the range 0–5 A. |
| FR-002 | The system shall convert the raw sensor voltage into current expressed in amperes. |
| FR-003 | The system shall retain access to raw ADC measurements for validation and debugging. |
| FR-004 | The system shall calculate average current. |
| FR-005 | The system shall determine peak current. |
| FR-006 | The system shall detect motor startup/inrush behaviour. |
| FR-007 | The system shall detect sustained overcurrent. |
| FR-008 | The system shall detect a motor stall/jam condition. |
| FR-009 | The system shall provide configurable warning and fault thresholds. |
| FR-010 | The system shall timestamp measurement data. |
| FR-011 | The system shall communicate measurement and diagnostic data through UART. |
| FR-012 | The final demonstrator shall implement CAN communication. |
| FR-013 | RS-485 shall be implemented as an additional industrial-interface experiment. |
| FR-014 | The system shall provide a diagnostic state such as NORMAL, WARNING or FAULT. |
| FR-015 | The system shall support offset and gain calibration. |
| FR-016 | Measurement data shall be exportable for MATLAB analysis. |
| FR-017 | A later revision shall provide an actuator shutdown output when a critical fault is detected. |

6. Preliminary Performance Requirements

These values are engineering targets, not yet frozen.

| ID | Parameter | Preliminary target |
| --- | --- | --- |
| PR-001 | Current range | 0–5 A |
| PR-002 | Reported current resolution | ≤10 mA |
| PR-003 | Room-temperature system accuracy | ≤±0.10 A |
| PR-004 | Measurement bandwidth | TBD during Phase 1 |
| PR-005 | ADC sampling rate | derived from required bandwidth |
| PR-006 | Stall detection time | TBD experimentally |
| PR-007 | Overcurrent detection time | TBD |
| PR-008 | Startup current capture | required |
| PR-009 | Continuous acquisition | required |

Bandwidth and sample rate will not be selected arbitrarily. They will be derived from the motor-current phenomena that the system must detect.

7. Diagnostic Concept

The system will eventually implement a state model similar to:

┌──────────┐

│   OFF    │

└────┬─────┘

│ Start

▼

┌───────────┐

│  STARTUP  │

└─────┬─────┘

│

Current settles

▼

┌────────┐

│ NORMAL │

└──┬───┬─┘

│   │

high load│   │ severe abnormality

▼   ▼

┌────────┐   ┌───────┐

│WARNING │──►│ FAULT │

└────────┘   └───────┘

Exact thresholds shall be determined from measured motor behaviour rather than guessed beforehand.

8. Validation Philosophy

For each requirement we will ultimately specify:

Requirement → Test method → Measurement → Pass criterion → Result

Example:

Requirement:

System shall detect sustained motor stall.

Test:

Mechanically prevent shaft rotation under controlled conditions.

Measurement:

Current waveform + diagnostic state + detection latency.

Pass:

FAULT generated within the specified time.

Result:

To be measured.

This traceability will be maintained throughout the project.

9. Phase 1 Items Still to Define

Before Phase 1 is complete we must determine:

- Exact target motor/actuator class

- Motor supply-voltage range

- What current phenomena we need to observe

- Required measurement bandwidth

- Required ADC sampling frequency

- Accuracy requirement

- Definition of overload

- Definition of stall

- Required fault-detection latency

- Protection/shutdown philosophy

- Environmental operating conditions

- Final verification matrix

No numerical threshold will be frozen without an engineering reason.